The Amoeba V2 Operation Code Set

| 0000 | LDA | Load the Accumulator with the contents of the addressed location. |
| --- | --- | --- |
| 0001 | STA | Store the contents of the Accumulator in the addressed location. |
| 0010 | LDAN | Load the Accumulator with the actual value of the address. |
| 0011 | ADD | Add the contents of the addressed location to whatever values in the Accumulator. (The result is in the Accumulator). |
| 0100 | SUB | Subtract the contents of the addressed location to whatever value is in the Accumulator. (The result is in the Accumulator). |
| 0101 | MLT | Multiply the contents of the addressed location by whatever value is in the Accumulator. (The result is in the Accumulator). |
| 0110 | DIV | Divide the contents of the addressed location into whatever value is in the Accumulator. (The result is in the Accumulator). No remainder |
| 0111 | JF | Jump forward a number of instructions. The number is given by the contents of the addressed location. |
| 1000 | JB | Jump back a number of instructions. The number is given by the contents of the addressed location. |
| 1001 | JFE | If the contents of the accumulator is zero, then jump forward a number of instructions. The number is given by the contents of the addressed location |
| 1010 | JBE | If the contents of the accumulator is zero then jump back a number of instructions. The number is given by the contents of the addressed location. |
| 1011 | CLR | Clear the Output |
| 1100 | OUTN | Display the contents of the Accumulator in the Output as a number. |
| 1101 | OUTC | Display the contents of the Accumulator in the Output as a character. |
| 1110 | NOP | No Operation. |
| 1111 | END | End program execution. |